電腦網路與通訊基礎 Homework #4

- [P7-21] (20%) An ISP is granted the block 80.70.56.0/21.  The ISP needs to allocate addresses for two organizations each with 500 addresses, two organizations each with 250 addresses, and three organizations each with 50 addresses.

- Find the number and range of addresses in the ISP block. (5%)

- Find the range of addresses for each organization and the range of unallocated addresses. (10%)

- Show the outline of the address distribution and the forwarding table. (5%)

- [P7-23] (10%) Can router R1 in Figure 7.35 receive a packet with destination address 140.24.7.194?  What will happen to the packet if this occurs?

- [P7-24] (10%) Assume router R2 in Figure 7.35 receives a packet with destination address 140.24.7.42.  How is the packet routed to its final destination?

- [P7-25] (10%) In an IPv4 datagram, the value of the total-length-field is (00A0)16 and the value of the header-length(HLEN) is (5)16.  How many bytes of payload are being carried by the datagram?  What is the efficiency (ratio of the payload length to the total length) of this datagram?

- [P7-26] (10%) An IP datagram has arrived with the following partial information in the header (in hexadecimal):

45000054  00030000  2006…

- What is the header size?

- Are there any options in the packet?

- What is the size of the data?

- Is the packet fragmented?

- How many more routers can the packet travel to?

- What is the protocol number of the payload being carried by the packet?

- [Chap8] (20%) Using Dijktra’s algorithm, create the shortest path tree and the forwarding table for node B (i.e. B is the source node) in the following figure.  You should show the least-cost tree in each iteration of the algorithm.

- [P8-5] (20%) In distance-vector routing, good news (decrease in a link metric) will propagate first.  In other words, if a link distance decreases, all nodes quickly learn about it and update their vectors.  In the following figure, we assume that a four-node internet is stable, but suddenly the distance between nodes A and D, which is currently 6, is decreased to 1 (probably due to some improvement in the link quality).  Show how this good news is propagated, and find the new distance vector for each node after stabilization. ‘